| BSL3 Rhapsody Protocol Day 1 8-Lane W/A/S/TB Day 1: Load cartridge, Lyse & capture cells, cDNA/Template Switch Prep Outside BSL3 (for 1 lane) - Place everything on ice, transfer to cold block when entering - Cell Capture Beads: Place on magnet, remove bead resus buffer, replace with 380uL cold sample buffer. Keep on ice - Prepare Lysis buffer: aliquot 2mL Lysis buffer + 10uL 1M DTT. Invert and keep on ice - Calcein & DRAQ7 aliquots - SMK Ab cocktail: (for 1E6 cells) - MHC1 SMK: 10uL + 1uL CD16/32 + 189uL Stain Buff - CD45 SMK: 10uL + 1uL CD16/32 + 199uL Stain Buff - Pipet mix, do not vortex! - Abseq cocktail: (for 1E6 cells) - 5 uL Fc block + 1 uL each AbO (x27) + 168uL Stain Buff - Pipet mix, do not vortex! | Rhapsody Cart Load & Start - Minimize time between cell pool and single cell capture - Calculate pooling volumes for each sample - Pipette mix gently then pool. Add Sample Buffer up to 340uL Pipette; Material; Volume (uL); Pipette Mode. P1200M; Air; 500; Prime/Treat. Set to Cell Load mode, manually pipette mix cells. P1200M; Pooled cells; 340; Cell Load. With cartridge on Express, Load Cells: - Incubate cart @ RT 15mins Counting Cart w/ Microscope - Parafilm cart openings, decon and move to microscope - Center using fiducial (post). Take note of number of wells in FOV - note: if the count of CAlcein positive cells is <100, count cells in another field of view until Calcein count is >= 100 - Viability=# Calcin pos(# Calcein neg+ # PI pos) - Estimate cell load. Total wells ~270,000 Load Cell Capture Beads - Place cart on Express Pipet; Material; uL; Pipet Mode. P1200M; Air; 500; Prime/Treat. Manually pipette mix beads first. P1200M; Capture Beads; 340; Bead Load. Bead Load: - Gently rock cartridge briefly to increase bead load yield - Incubate cart @RT 3mins Pipet; Material; uL; Pipet Mode. P1200M; Air; 500; Wash. P1200M; Cold Sample Buffer; 500; Wash. P1200M; Air; 500; Wash. P1200M; Cold Sample Buffer; 500; Wash. Bead Wash: - Optional count to check number of beads+cells Lyse Cells & Bead Retrieval - Incubate @RT 2 mins. Maintain recommended lysis time for best performance - move front slider to BEADS and set pipet to retrieval mode - At 2 mins end IMMEDIATELY slide magnet onto cart. Leave for exactly 30 sec - Towards end of 30sec take up ~1mL Lysis Buffer - After 30s, remove magnet and add Lysis - Move front slider to OPEN and remove collection tube - Proceed to Wash Cell Capture Beads - Transfer retrieved volume to screwcap tube. Parafilm and decon for transfer out of BSL3 - Discard waste container. Clean and decon HT Express |
| --- | --- |
| IN BSL3 - Transfer reagents to cold block Cartridge Prime & Treat: - Open slider and check waste, load new collection tube - Prime cartridge and leave for 1 min RT - Treat cartridge and use within 4 hours AbSeq & Sample Tag Stain - Aliquot 5E5-1E6 cells, spin down 400g 5mins 4C - Resuspend in 200uL SMK Ab cocktail. Incubate @RT 20mins - Add ~500uL Sample buffer to resuspend - Spin 400g 5mins, remove SN carefully - Resuspend in 200uL Ab cocktail. Incubate on ice 30mins - Add 1mL BD stain buffer and pip mix - Spin 400g 5mins, remove SN carefully - Resuspend 1mL Staining Buffer, 10uL cell count, no dilution - Pool SMK’d samples for total of 150-200k cells, spin and carefully remove SN - Resus in 620uL cold Sample Buffer Cell Viability Stain - Add 3.1 μL of 2 mM Calcein AM and 3.1 μL of 0.3 mM DRAQ7™, (1:200 dilution) gently pipet mix - Incubate IN DARK @37°C - Strain cells thru flow tube cap and immediately count Keep cells in dark on ice! |  |

| Outside BSL3 Wash Cell Capture Beads - Place 5mL tube on magnet for 1 min. Remove 4mL SN (leave 1mL) - Gently pipet mix and transfer to 1.5mL LoBind tube - Can rinse with small volume Lysis Buffer - Place on magnet for ≤2 mins. Remove all SN. AVOID BUBBLES - Off magnet, add 1mL cold Bead Wash Buffer and mix - Repeat wash for total of 2 cycles. Keep in buffer 2nd time - Place on ice, start reverse transcription ≤30mins after wash cDNA Synthesis & Template Switch - Set thermomixer to 42°C cDNA/temp switch mix; 1X (uL); 1X+10% overage (uL). RT Buffer; 40; 44. dNTP; 20; 22. RT 0.1 M DTT; 10; 11. Bead RT/PCR Enhancer; 12; 13.2. Rnase Inhibitor; 10; 11. Reverse Transcriptase; 10; 11. Nuclase-free H2O; 98; 107.8. Total:; 200; 220. Make cDNA/template switching MM keep on ICE - Place 1.5mL beads on magnet >2 mins. Remove SN. - Add 200uL cDNA MM. Pipet mix and transfer to new tube - Incubate on thermomixer 1200rpm @42°C for 30mins - During last 2mins make TSO mix: 6.3uL TSO + 2.1uL 1 M MgCl2 = 8.4uL total - Once 30mins is up, add 8uL TSO mix and gently pipet mix. - Incubate on thermomixer again 1200rpm @42°C for 30mins - Place on magnet until clear, discard SN - Remove from magnet and gently resus in 75uL Elution Buffer. DO NOT VORTEX! Stopping point: can store Capture Beads up to 4-7 days at 2-8°C DO NOT FREEZE CAPTURE BEADS |  |
| --- | --- |