Dual Index PCR of cDNA RNAseq Library

| Forward Primer | i5 index name | Sequence for Manifest |
| --- | --- | --- |
| 1 | TruSeq D501 | TATAGCCT |
| 2 | TruSeq D502 | ATAGAGGC |
| 3 | TruSeq D503 | CCTATCCT |
| 4 | TruSeq D504 | GGCTCTGA |

| Reverse Primer | i7 index name | Sequence for Manifest |
| --- | --- | --- |
| 1 | Nextera N709 | GCTACGCT |
| 2 | Nextera N710 | CGAGGCTG |
| 3 | Nextera N711 | AAGAGGCA |
| 4 | Nextera N712 | GTAGAGGA |
| 5 | TruSeq D705 | ATTCAGAA |

- Prep:

  - Make ~5mL of 80% ethanol

  - Take AMPure Beads, DS5000 TapeStation Reagents, PCR MM, forward/reverse primers, Nuclease-free water, and Elution buffer out to equilibrate to RT

- If cDNA not QC’d yet, run Qubit, dilute to 1ng/uL, and TapeStation:

  - Ladder Tube: 2uL Sample Buffer + 2uL Ladder

  - Sample Tubes: 2uL Sample Buffer + 2uL Sample at 1ng/uL

  - Vortex ~30 sec and spin down briefly, protect from light. Incubate 2 mins.

  - Bring tubes, tips, and strip tape to TapeStation machine. Set up and start run. Record molar concentration of desired product range

- Make index PCR mix:

| RNAseq index PCR Mix | 1X+10% overage (uL) |
| --- | --- |
| PCR MM | 27.5 |
| Library Forward Primer | 5.5 |
| Library Reverse Primer | 5.5 |
| Nuclease-free H2O | 5.5 |
| Total | 44 |

- Dilute RNAseq library concentration to 2nM based on TapeStation metrics. If concentration <2nM do not dilute.

- In PCR strip tube add 40uL index PCR Mix + 10uL 2nM RNAseq library. Pipet mix 10x

- Program Thermocycler:

| RNAseq index PCR | Cycles | Temp | Time |
| --- | --- | --- | --- |
| Hot Start | 1 | 95°C | 3 min |
| Denaturation | *see table to right | 95°C | 30 sec |
| Annealing |  | 60°C | 30 sec |
| Extension |  | 72°C | 30 sec |
| Final Extension | 1 | 72°C | 1 min |
| Hold | 1 | 4°C | ∞ |

| Concentration of RNAseq Library | Recommended Cycles |
| --- | --- |
| <2nM | 9 |
| 2nM | 8 |

- RNAseq index 0.8X PCR cleanup with AMPure Beads:

  - Spin down PCR tubes

  - Vortex AMPure Beads for ~1 min

  - Check PCR product volume is exactly 50uL, if not top up with water. Or calculate bead to product ratio 0.8.

  - Add 40uL of AMPure beads (0.8X) to PCR tube. Pipet mix 10x. Incubate 5mins.

  - After incubation, place on strip tube magnet for 5 mins. Discard supernatant

  - Gently add 200uL 80% ethanol. Wait 30 sec and discard supernatant.

  - Repeat step f. for total of 2 washes

  - Air dry for ~1 min or until bead pellet looks matte (Do not overdry until cracked)

  - Take off magnet and resuspend with 40uL Elution Buffer. Incubate 2 mins at RT

  - Spin down briefly and place on magnet until clear

  - Pipet ~40uL eluate into new 1.5mL tube

  - Ready for Qubit and Tapestation